# TEAM DESCRIPTION PAPER

Team Tux_3 (from left to right: Alexander Belov, Toni Schürmann, Max Harting, Henning Baumgartner

Email: tux3.gr@protonmail.com

YouTube: https://www.youtube.com/@Tux_3_gr

GitHub: https://github.com/alexander0038/TDP-Robocup-Tux_3/

## Short Team Description

We, Tux_3, compete in the RoboCupJunior Soccer Entry League and were formed in 2026 by merging two previously independent teams.

In the 2025 season, the members competed separately as Tux_3 (Alexander and Henning) and Tux_4 (Max and Toni), both using LEGO-based robots. At the Berlin qualification tournament, Tux_3 achieved 2nd place and Tux_4 achieved 4th place. Both teams qualified for the German Championship in Nuremberg, where Tux_3 finished 8th (one match away from European qualification) and Tux_4 finished 13th.

In 2026, the two teams combined their experience and formed a single team under the name Tux_3, redesigning the robot with fully custom 3D-printed components. At the Berlin qualification tournament 2026, the new team achieved 2nd place.

## Short Robot Description

The Tux_3 robot is a custom-built RoboCupJunior Entry League soccer robot designed for reliability, robustness, and continuous improvement. After competing with LEGO-based robots in 2025, the team fully redesigned the system for the 2026 season using custom 3D-printed components and a self-developed electronic architecture.

The robot is based on a two-motor differential drive system with integrated ball control and goal detection mechanisms. The overall design focuses on mechanical stability, precise ball handling, and structured decision-making during matches.

## Hardware

Drive System

- 2× Pololu 25Dx48L MP 12V motors

- Differential drive configuration

- Infineon IFX9201SG motor drivers

Ball Handling

- GA16Y-050-CE motor for dribbler

- Infineon BTN8982TA driver for kicker

- Light barrier for ball possession detection

Sensors

- Vishay TSSP6038TR infrared sensors for ball detection

- CMPS14 compass module for orientation stabilization

- Pixy2 camera for goal detection

- DFRobot_URM09 Ultrasonic sensors for distance measurement and wall correction

Electronics

- ESP32 microcontroller

- Custom wiring and structured component layout

- 3D-printed chassis designed in Autodesk Fusion

More about Hardware in ‘/Hardware/README.md’

## Software

The software is structured around a high-frequency main control loop running on the ESP32. The system separates sensor acquisition, decision logic, and motor control into modular components. Our program is based on the CoRoSoN-Kit (https://github.com/ItzJannis/CoRoSoN-Package_Draft).

Key characteristics:

- State-based decision logic:

- Ball tracking using infrared sensors

- PID Control:

- Goal alignment using camera and compass

- Custom stuck-detection and recovery behavior:

- Wall correction using ultrasonic feedback

The software was developed with a focus on maintainability, clarity, and match robustness.

More about the software in: https://github.com/alexander0038/TDP-Robocup-Tux_3/tree/main/software

## Challenges During the Season

During the transition from LEGO to a fully custom 3D-printed robot, several challenges were encountered:

- Mechanical Stability:
 Early prototypes suffered from structural weaknesses and misalignment in the dribbler system.

- Different Programming Languages:
 We started with the programming language NXC, which translates to “Not exactly C”, and later switched to C/C++.

- Sensor Calibration:
 Reliable ball detection and consistent compass readings required extensive testing and calibration.

- Differential Drive Precision:
 Achieving accurate turning and goal alignment without omni wheels required significant tuning of motor control.

## Challenges Successfully Mastered

- Structural redesign improved mechanical robustness.

- Stable ball control was achieved through optimized dribbler positioning.

- Sensor placement and filtering significantly increased detection reliability.

- Stuck-detection logic improved match recovery performance.

- Team collaboration improved after merging two former teams into one unified structure.

These improvements led to a 2nd place finish at the 2026 Berlin qualification tournament.

## Future Plans

For the next season, the team plans to:

- Compete in Robocup Junior Soccer Lightweight.

- Learn more about the hardware.

- Deepen our knowledge about programming

- Improve our ability to analyze and find mistakes in hardware and software faster

- Improve teamwork even further (if ’s even possible)

We hope that one day we will compete in the World Championship and win first place. What sounds like a dream today might come true one day.